UHF Even

Reader

User manual V1.0

Table of contents

1Docking communication protocol	2

1.1 Serial communication parameters	2

1.2 data frame format	2

2 command frame set	3

2.1 list of commands	3

2.2 Module general control commands	4

2.2.1 RFM_MODULE_INT（Initialize the device）	4

2.2.2 RFM_REBOOT（reset）	5

2.2.3 RFM_SET_PWR	 (Set RF output power)	5

2.2.4 RFM_SET_GET_RFIDPRO (Set/Read RF protocol standards supported by the module)	5

2.2.5 RFM_SET_GET_NETPARA (Set/read device network port information)	6

2.2.6 RFM_SET_GET_REMOTE_NETPARA（Set/read device remote network port information）	8

2.2.7 RFM_GET_DEVICEINFO（Get module information）	9

2.2.8 RFM_SET_ALL_PARAM (Set all configurable parameters)	10

2.2.9 RFM_GET_ALL_PARAM (Read all configurable parameters)	14

2.2.10 RFM_SET_GET_PERMISSION_PARAM (Set read permission)	15

2.2.11 RFM_SET_GET_IOPUT_PARAM (Set read input and output control)	16

2.2.12 RFM_SET_GET_WiFi_PARAM (Set/read WiFi information)	17

2.2.13 RFM_SET_GET_S-PERMISSION_PARAM (Set read permission)	19

2.3 International standard (ISO 18000-6C) protocol related commands	20

2.3.1 RFM_INVENTORYISO_CONTINUE( tag inventory)	20

2.3.2 RFM_INVENTORY_STOP (stop counting)	21

Appendix A. Operational Status Returned by the Tag	23

Appendix B. CRC16 check reference C code	24

Appendix C. Definition of STATUS	25

1Docking communication protocol

This module can communicate with external applications through serial port RS232/TTL, RS485 and network port RJ45.

1.1 Serial communication parameters

The module communicates with external applications through the serial port

Table A-1 Module docking serial port parameters

| project | parameter | Remark |
| --- | --- | --- |
| baud rate | 115200bps |  |
| data bits | 8 |  |
| stop bit | 1 |  |
| parity bit | None |  |

1.2 Data frame format

The data packets sent by the upper-layer application are hereinafter referred to as "commands", and the data packets returned by the module to the upper-layer applications are hereinafter referred to as "responses". All the following data segments are in bytes in length.

Data transmission sequence: For each data item consisting of multiple bytes, the most significant byte is sent first, and the least significant bit is sent first in the byte.

The data frame format of the command is shown in Table A-2:

Table A-2 Data Frame Format for Commands

| MSB |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- |
|  | Control Field |  |  | Information Field | EpLENogue Field |
| HEAD | ADDR | CMD | LEN | Data[] | CHECK |
| 1Byte | 1Byte | 2Bytes | 1Byte | 0~255Byte | 2Byte |

The data frame format of the response is shown in Table A-3:

Table A-3 Response data frame format

| MSB |  |  |  |  |  | LSB |
| --- | --- | --- | --- | --- | --- | --- |
|  | Control Field |  |  | Information Field |  | Verify Field |
| HEAD | ADDR | CMD | LEN | STATUS | Data[] | CHECK |
| 1Byte | 1Byte | 2Bytes | 1Byte | 1Byte | 0~254Byte | 2Byte |

The data frame structure includes the following three parts: frame control field (Control Field), information data field (Information Field) and frame end field (Verify Field).

|  |  | length | illustrate |  |
| --- | --- | --- | --- | --- |
| Control Field | HEAD | (byte) | HEAD is fixed to hexadecimal number 0xCF, this byte is used for data frame synchronization. |  |
|  | ADDR | 1 | Reader address. Address range: 0x00~0xFE, 0xFF is the broadcast address, the reader only responds to the command with the same address as its own and the address is 0xFF. The address of the reader is 0x00 when it leaves the factory. |  |
|  | CMD | 1 | Command code, command word definition see Table A-7. |  |
|  | LEN | 2 | The length is the data length of all bytes of the actual Information Field, and the maximum data length is 255Bytes (0xFF). |  |
| Information Field | STATUS | 1 | STATUS is the execution status of the upper computer command contained in the module's response. STATUS only exists in the response data frame returned by the module, and there is no STATUS part in the command frame sent by the upper-layer application. A STATUS of 0 indicates that the command was successfully executed. STATUS is defined in Table A-6. |  |
|  | Data[] | 1 | parameter field. In the actual command, it can be absent. Data[] is the actual data to be transmitted, and the valid bytes of Data[] will be specifically defined in each command format. |  |
| Verify Field | MSB-CRC16 | indefinite | CRC16 high byte. | CRC16 is the CRC16 value of all data from HEAD to Data[]. See Appendix B for the CRC16 reference code. |
|  | LSB-CRC16 | 1 | CRC16 low byte. |  |

2 command frame set

The UHF RFID reader module is simply referred to as "module" in the following description.

2.1 list of commands

Table A-7 Example List of Control Commands

| command name | CMD control byte | Function description | Remark |
| --- | --- | --- | --- |
| EPC Directive |  |  |  |
| RFM_INVENTORYISO_CONTINUE | 0x0001 | Tag inventory (process) |  |
| RFM_INVENTORYISO_STOP | 0x0002 | stop label inventory |  |
| Module custom directive |  |  |  |
| RFM_MODULE_INT | 0X0050 | Initialize the device |  |
| RFM_REBOOT | 0x0052 | reset | Restore the default value of the basic parameter |
| RFM_SET_PWR | 0x0053 | Set RF power | 0-30dBm |
| RFM_SET_GET_RFIDPRO | 0x0059 | Set/Read RF Protocol | Currently the device only supports |
| RFM_SET_GET_NETPARA | 0X005F | Set/read network port information | ISO 18000-6C |
| RFM_SET_GET_REMOTE_NETPARA | 0x0064 | set get set remote network port information | Suitable for LAN |
| RFM_GET_DEVICEINFO | 0X0070 | Get all module version numbers and sn codes | Applicable to the whole network, remote across network segments |
| RFM_SET_ALL_PARAM | 0x0071 | Set all basic parameters |  |
| RFM_GET_ALL_PARAM | 0x0072 | Read all base parameters | Pack all basic parameters and set them all at once |
| RFM_SET_GET_PERMISSION_PARAM | 0x0073 | Read permission settings |  |
| RFM_SET_GET_IOPUT_PARAM | 0x0074 | Input and output IO settings |  |
| RFM_SET_GET_WiFi_PARAM | 0x0075 | Set/read WiFi information |  |
| RFM_SET_GET_S-PERMISSION_PARAM | 0X0076 | Specific read permission settings |  |

2.2 Module general control commands

2.2.1 RFM_MODULE_INT (initializing the device)

Confirm whether the device is online, use when the host computer is connected to the device

- command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0050 | 0x00 | 2Byte |

- Response format and status bytes

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0050 | 0x01 | 1Byte | 2Byte |

 STATUS：This command will only return successful execution (value 0x00), other values are invalid；

2.2.2 RFM_REBOOT（reset）

Restore factory settings command, when the command is successfully executed, the basic parameters of the module are restored to factory settings

- Command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0052 | 0x00 | 2Byte |

- Response format and status bytes

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0052 | 0x01 | 1Byte | 2Byte |

 STATUS：0x00The execution is successful, other values are shown in Table A-6;

2.2.3 RFM_SET_PWR	 (Set RF output power)

This command is used to set the RF output power of the module. The user needs to use this command to set the RF output power of the module before using the module to operate the tag. If the user does not set the RF output power of the module, the module will use the default settings when working.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Power | Resv |  |
| 0xCF | 0xFF | 0x0053 | 0x02 | 1Byte | 1Byte | 2Byte |

  - Power：Output power, the unit is: dBm, the value range is: [0, 30]dBm, the default is 26.

  - Resv：System reserved field, default 0x00;

- Response format and status bytes

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0053 | 0x01 | 1Byte | 2Byte |

 STATUS：0x00The execution is successful, other values are shown in Table A-6;

2.2.4 RFM_SET_GET_RFIDPRO (Set/Read RF protocol standards supported by the module)

This command sets/reads the RF RFID protocol standard specification of the module (by issuing this command, the protocol standard can be set to determine whether the module is successfully connected).

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | RFIDPRO |  |
| 0xCF | 0xFF | 0x0059 | N Byte | 1Byte | NC or 1Byte | 2Byte |

  - Option：command control options

0x01：Set, followed by RFIDPRO of 1Byte length；

0x02：Read, then do not connect RFIDPRO；

other values: invalid;

  - RFIDPRO：Protocol options, see the table below for details。

- Response format and status bytes

Set response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0059 | 1Byte | 1Byte | 1Byte | 2Byte |

Get the response:

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | RFIDPRO |  |
| 0xCF | 0x00 | 0x0059 | 3Byte | 1Byte | 1Byte | 1Byte | 2Byte |

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00 is executed successfully, other values are shown in Table A-6; |
| Option | 1 | 0x01: set; 0x02: get |
| RFIDPRO | 1 | 0x00: ISO 18000-6C; 0x01: GB/T 29768; 0x02: GJB 7377.1; currently only ISO 18000-6C is supported. |

2.2.5 RFM_SET_GET_NETPARA (set/read device network port information)

- This command sets/reads the network port information of the module. The default setting is TCP Server. The host computer connection needs to set the IP address and port of the corresponding device.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | NetPara |  |
| 0xCF | 0xFF | 0x005F | 1 or 2 Byte | 1Byte | NC or 1Byte | 2Byte |

Option：Command control options

0x01: set;

0x02: read;

Other values: invalid；

Response format and status byte

set response：

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x005F | 1Byte | 1Byte | 1Byte | 2Byte |

Get response：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x00 | 0x005F | 2+N Byte | 1Byte | 1Byte | N Byte | 2Byte |

| field |  | byte | Describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00 is executed successfully, other values are shown in Table A-6; |
| Option |  | 1 | 0x01: set; 0x02: get |
| NetPara | IPAddr | 4 | Device IP address, if the IP address is 192.168.1.1, the data is [0xC0,A8,0x01,0x01] |
|  | MacAddr | 6 | Mac address, if multiple devices are working on the same site, please use different MAC addresses; For example: mac[6]={0x00,0x08,0xdc,0x11,0x11,0x11}, the mac address is 00-08-DC-11-11-11 |
|  | Port | 2 | Listening port, the value is [0,65536], the default is 2022; |
|  | NetMask | 4 | Subnet mask, default is [0xFF,0xFF,0xFF,0x00] |
|  | GateWay | 4 | Default gateway, default is [0xC0, 0xA8, 0x01, 0x01] |

2.2.6 RFM_SET_GET_REMOTE_NETPARA（Set/read device remote network port information）

This command sets/reads the remote network port information, the device sends data to the remote IP, and the remote computer can monitor and receive data.

- Command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | NetPara |  |
| 0xCF | 0xFF | 0x0064 | 1 or 2 Byte | 1Byte | NC or 1Byte | 2Byte |

- Option: command control option

- 0x01: set;

- 0x02: read;

- other values: invalid;

Response format and status bytes

Set response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0064 | 2Byte | 1Byte | 1Byte | 2Byte |

Get the response:

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x00 | 0x0064 | 2 + N Byte | 1Byte | 1Byte | NC or N Byte | 2Byte |

| field |  | byte | Describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00 is executed successfully, other values are shown in Table A-6; |
| Option |  | 1 | 0x01: set; 0x02: get |
| NetPara | Enable | 1 | Enable switch, 1: enable transmission, 0: disable transmission. |
|  | IPAddr | 4 | Remote IP address, if the IP address is 192.168.1.1, the data is [0xC0, 0xA8, 0x01, 0x01] |
|  | Port | 2 | Listening port, the value is [0,65536], the default is 5000； |
|  | HeartTime | 1 | Heartbeat packet time, to prevent disconnection, unit*5S |

2.2.7 RFM_GET_DEVICEINFO（Get module information）

This command is used to obtain the version information of the current device, including the hardware version number, firmware version number and SN number of the CP module and RFID module.

- Command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0070 | 0x00 | 2Byte |

- Response format and status bytes

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | HardVer | FirmVer | SN_code |
| 0xCF | 0x00 | 0x0070 | 0x99 | 1Byte | 32Bytes | 32Bytes | 12Bytes |

| PAYLOAD |  |  | CHECK |
| --- | --- | --- | --- |
| Reserve-I | Reserve-II | Reserve-III |  |
| 32Bytes | 32Bytes | 12Bytes | 2Byte |

Definition of each byte:

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | This command will only return successful execution (value 0x00), other values are invalid. |
| HardVer | 32 | Module hardware version number, the format is ASCII code; the hardware version number is less than 32 bytes, and the remaining bytes are supplemented with '\0'. |
| FirmVer | 32 | Module firmware version number, the format is the same as the hardware version number. |
| SNCode | 12 | Module 12-byte serial number in ASCII format. |
| Reserve-I | 32 | spare |
| Reserve-II | 32 | spare |
| Reserve-III | 12 | spare |

2.2.8 RFM_SET_ALL_PARAM (Set all configurable parameters)

This command is used to set all configurable parameters of the device. The user needs to use this command to set all the configurable parameters of the device before using the module to operate the label. If the user does not make this setting, the device will use the default setting when working.

- Command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Addr | RFIDPRO | Work Mode | Interface | Baudrate | WGSet | Ant | RfidFreq |
| 0xCF | 0xFF | 0x0071 | N Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 8Bytes |

| PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfidPower | InquiryArea | QValue | Session | AcsAddr | AcsDataLen | FilterTime | TriggerTime | BuzzerTime | Polling Interval |  |
| 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes |

  - Addr: The communication address of the device, the default is 0x00. This address cannot be 0xFF. If set to 0xFF, the read/write module will return parameter error information。

  - RFIDPRO: Protocol standard specification for equipment radio frequency RFID, 0x00: ISO 18000-6C; 0x01: GB/T 29768; 0x02: GJB 7377.1; currently only supports ISO 18000-6C

  - WorkMode: The working mode of the device, the default value is 0.。

| WorkMode | Working mode |
| --- | --- |
| 0 | Answer mode |
| 1 | Active mode |
| 2 | Trigger mode |

  - Interface：The communication interface of the device, the default value is 0x80, the specific definition is as follows:

| Interface | Interface Type |
| --- | --- |
| 0x80 | RS232 |
| 0x40 | RS485 |
| 0x20 | RJ45 |
| 0x10 | WiFi |

  - Baudrate：Serial port baud rate, the default value is 4, the specific interpretation is as follows:

| baudrate | actual baud rate |
| --- | --- |
| 0 | 9600bps |
| 1 | 19200 bps |
| 2 | 38400 bps |
| 3 | 57600 bps |
| 4 | 115200 bps |

  - WGSet：The configuration parameters of the Wiegand data output interface, the default value is 0x00, and the specific definitions are as follows:

| WGSet | Bit7 | Bit6 | Bit5 | Bit4 | Bit3 | Bit2 | Bit1 | Bit0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit Interpretation | 0: Disable Wiegand output | 0：wg26 1：wg34 | 0: LSB first | spare | Spare | Spare | Spare | Spare |

  - Ant：All the antenna numbers of the device, the bit indicates the selected antenna, the corresponding Bit value of 1 indicates that the antenna is used, and the value of 0 indicates that the antenna is not used; starting from the low bit, the 0th bit indicates the No. The bit represents the No. 2 antenna, and so on, it can represent up to 8 antennas; different modules support different antennas, depending on the specific situation; the default value is 0x01, which means the No. 1 antenna.

  - RfidFreq：The RFID frequency related parameters of the device are used to select the frequency band and the upper limit frequency point and the lower limit frequency point in each frequency band. The length is 8Bytes. The specific parameters are as follows：

| RfidFreq |  |  |  |  |
| --- | --- | --- | --- | --- |
| REGION | STRATFREI | STRATFRED | STEPFRE | CN |
| 1 Byte | 2Byte | 2Byte | 2Byte | 1Byte |

Definition of each byte:

To set the frequency, first determine the frequency range according to REGION, and then refer to STRATFREI, STRATFRED, STEPFRE, CN to calculate the specific frequency range.

| field | byte | describe |
| --- | --- | --- |
| REGION | 1 | National frequency range: 0x00: Customized by users according to their needs; 0x01: US [902.75~927.25] 0x02: Korea [917.1~923.5] 0x03: EU [865.1~868.1] 0x04: JAPAN [952.2~953.6] 0x05: MALAYSIA [919.5~922.5] 0x06: EU3 [865.7~867,5] 0x07: CHINA_BAND1 [840.125~844.875] 0x08: CHINA_BAND2 [920.125~924.875] |
| STRATFREI | 2 | The integer part of the starting frequency in megahertz; such as 920.125MHz, STRATFREI = 920 = 0x0398, high byte = 0x03, low byte = 0x98; |
| STRATFRED | 2 | Fractional part of the starting frequency in megahertz; e.g. 920.125MHz, STRATFRED = 125, high byte=0x00，低字节=0x7D |
| STEPFRE | 2 | Frequency step (KHz), please refer to the calculation formula of each frequency band; for example, 125KHz, STEPFRE = 125, high byte = 0x00, low byte = 0x7D; |
| CN | 1 | number of channels; |

Frequency calculation formula: Minimum frequency point: Fmin = STRATFREI + STRATFRED/1000 (unit: MHz)

Maximum frequency point: Fmax = Fmin + STEPFRE*CN/1000 (unit: MHz)

Calculation formula for each frequency band: (The frequency range set must be within the national standards)

Chinese band2: Fs = 920.125 + CN * 0.25 (MHz) where CN∈[0, 19].

US band: Fs = 902.75 + CN * 0.5 (MHz) where CN ∈ [0,49].

Korean band: Fs = 917.1 + CN * 0.2 (MHz) where CN∈[0, 31].

EU band: Fs = 865.1 + CN*0.2(MHz) where CN∈[0, 14].

Ukraine band: Fs = 868.0 + CN*0.1(MHz) where CN∈[0, 6].

Peru band: Fs = 916.2 + CN*0.9(MHz) where CN∈[0, 11].

Chinese band1: Fs = 840.125 + CN * 0.25 (MHz) where CN∈[0, 19].

EU3 band: Fs = 865.7 + CN * 0.6(MHz) where CN ∈ [0, 3].

US band3: Fs = 902 + CN * 0.5 (MHz) where CN ∈ [0,52].

Taiwan band: Fs = 922.25 + CN * 0.5 (MHz) where CN ∈ [0,11].

  - RfidPower: The RFID output power of the device, the unit is dBm, the value range is: [0, 30]dBm, other values are invalid.

  - InquiryArea: The storage area where the device wants to access the tag. 0x00: reserved area; 0x01 (default): EPC storage area; 0x02: TID storage area; 0x03: user storage area; other values are reserved. If other values are present in the command, a parameter error message is returned.

  - QValue: The initial Q value used when querying the EPC tag. The Q value should be set so that the number of tags in the field is approximately equal to 2Q. The default value of the Q value is 4, and the range of the Q value is 0 to 15. If other values appear in the command, a parameter error message will be returned.

- Session: The session value used when querying the EPC tag, the default value is 0, the value range is [0, 3], other values will return a parameter error message.

| Session | Notes |
| --- | --- |
| 0 | Session uses S0 |
| 1 | Session uses S1 |
| 2 | Session uses S2 |
| 3 | Session uses S3 |

  - AcsAddr: The starting address of the label storage area to be accessed by the device, unit: Byte, default value: 0x00: When accessing the EPC area, 0x00 indicates the starting address of the EPC number segment except the CRC and PC segments to access the EPC area; access other storage areas , 0x00 indicates the starting address of the memory area.

  - AcsDataLen：The data length of the tag storage area to be accessed by the device, unit: Byte, default value: 0x00.

  - FilterTime：Filter time, within the value time after successfully reading a tag data, the tags with the same data are filtered out. The unit is: S, the value range is: [0, 255], others are invalid; the default value is 0, no filtering.。

  - TriggerTime：The query duration after the device receives the trigger signal, the unit is S, the default value is 1, and the value range is: [0, 255], others are invalid.

  - BuzzerTime：The buzzer beeping time after the device is successfully executed, the unit is 10ms, the value range is: [0, 255], the others are invalid; the default is 1, when it is 0, it means that the buzzer does not beep.

  - PollingInterval：Query interval time, the unit is 10ms, the value range is: [0, 255], others are invalid, the default is 1.

- Response format and status byte

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0xXX | 0x0071 | 0x01 | 1Byte | 2Byte |

  - STATUS：

  - 0x00：The command was executed successfully;

  - 0x01：Parameter error;

  - Other values: invalid;；

2.2.9 RFM_GET_ALL_PARAM (Read all configurable parameters)

- This command is used to read the RF power currently set by the module. The user can use this command to check the power setting of the module before using the module to operate the label.

- command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0072 | 0x00 | 2Byte |

- Response format and status byte

| HEAD | ADDR | CMD | LEN | PAYLOAD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STATUS | Addr | RFIDPRO | Work Mode | Interface | Baudrate | WGSet | Ant | RfidFreq |
| 0xCF | 0xFF | 0x0072 | N Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 8Bytes |

| PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfidPower | InquiryArea | QValue | Session | AcsAddr | AcsDataLen | FilterTime | TriggerTime | BuzzerTime | Polling Interval |  |
| 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 2Bytes |

  - STATUS: 0x00 is executed successfully, other values are shown in Table A-6;

  - Other parameters refer to RFM_SET_ALL_PARAM

2.2.10 RFM_SET_GET_PERMISSION_PARAM (Set read permission)

This command is used to set password and mask to query permission parameters

- Command format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0073 | N Byte | 1Byte | 1Byte | 4Bytes | 1Byte | 1Byte | 1Byte | 12Bytes | 1Byte | 2Bytes |

  - Option: command control option, 0x01 is set, 0x02 is read, other values are invalid.

  - CodeEN: Password function enable parameter, length 1 byte. 0x01 is enabled, 0x00 is not enabled. Default 0x00.

  - Codes    ：The access password of the tag, with a length of 4 bytes. Default[0x00,0x00,0x00,0x00]

  - MaskEN：Mask function enable parameter, length 1 byte. 0x01 is enabled, 0x00 is not enabled. Default 0x00.

  - StartAdd：Mask start address, length 1 byte, unit byte. Default 0x00.

  - Mask Len：Mask length, length 1 byte, unit byte, maximum 12. Default 0x00.

  - MaskData：Mask data, the length is 12 bytes, when the mask length is less than 12 bytes, the following byte data is supplemented with 0.default[0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00]。

  - MaskCondition：Mask condition, length 1 byte. 0x00: Password or mask match; 0x01: Password and mask match at the same time. Default 0x00.

- Response format and status byte

- Set response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0073 | 2Byte | 1Byte | 1Byte | 2Byte |

- get response：

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0073 | N Byte | 1Byte | 1Byte | 1Byte | 4Byte | 1Byte | 1Byte | 1Byte | 12Bytes | 1Byte | 2Bytes |

  - STATUS: 0x00 is executed successfully, other values are shown in Table A-6;

2.2.11 RFM_SET_GET_IOPUT_PARAM (Set read input and output control)

- This command is used for relay usage parameters, buffer switch parameters and common protocol usage parameters.

- command format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Relay EN | Relay Time | Relay Power EN | Trig Way | Buffer EN | Protocol EN | Protocol Type | Protocol Format |  |
| 0xCF | 0xFF | 0x0074 | N Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 10 Bytes | 2Bytes |

  - Option：Command control options, 0x01 is set, 0x02 is read, other values are invalid.

  - RelayEN：Relay function enable parameter, length 1 byte. 0x01 is enabled, 0x00 is not enabled. Default 0x00。

  - RelayTime：Relay normally open pin conduction time parameter, length 1 byte, unit S. Default 0x03.

  - RelayPowerEN: The power enable parameter of the relay common terminal, the length is 1 byte. 0x01 turns on, 0x00 turns off. Default 0x00.

  - TrigWay: The effective trigger mode parameter of the trigger pin, the length is 1 byte. 0x01 high level (2V-5V) trigger, 0x00 low level (less than 0.5V) trigger. Default 0x01.

  - BufferEN：Cache function enable parameter, length 1 byte. 0x01 is enabled, 0x00 is not enabled. Default 0x00.

  - ProtocolEN：Specific data interaction function enable parameter, length 1 byte. 0x01 is enabled, 0x00 is not enabled. Default 0x00.

  - ProtocolType：Specific data exchange protocol type, length 1 byte. Default 0x00.

  - ProtocolFormat：Specific data exchange protocol format, length 10 bytes.

  - Default [0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x00].

- Response format and status bytes

- Set response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0074 | 1Byte | 1Byte | 1Byte | 2Byte |

- Get the response:

| HEAD | Control Field |  |  |  | PAYLOAD |  |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN |  | STATUS | OPTION | Relay EN | Relay Time | Relay Power EN | Trig Way | Buffer EN | Protocol EN | Protocol Type | Protocol Format |  |
| 0xCF | 0xFF | 0x0074 | N Byte |  | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 1Byte | 10 Bytes | 2Bytes |

  - STATUS：0x00 is executed successfully, other values are shown in Table A-6;

2.2.12 RFM_SET_GET_WiFi_PARAM (Set/read WiFi information)

This command sets/reads the WiFi information of the module, the network mode is TCP Client, and the corresponding remote IP address and port need to be set;

- Wi-Fi network configuration mode: send ssid and pwd for configuration, and the device will automatically connect if the configuration is successful.

- command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Option | WiFiPara |  |
| 0xCF | 0xFF | 0x0075 | 1Byte | 1Byte | NC or 1Byte | 2Byte |

- Option: command control option

- 0x01: set;

- 0x02: read;

- other values: invalid;

Response format and status byte

Set response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0075 | 1Byte | 1Byte | 1Byte | 2Byte |

Get the response:：

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Option | NetPara |  |
| 0xCF | 0x00 | 0x0075 | N Byte | 1Byte | 1Byte | NByte | 2Byte |

| field |  | byte | describe |
| --- | --- | --- | --- |
| STATUS |  | 1 | 0x00 is executed successfully, other values are shown in Table A-6; |
| Option |  | 1 | 0x01: set; 0x02: get |
| WiFiPara | WiFiEN | 1 | WiFi enable parameter, mode selection. bit0: 0: turn off wifi; 1: turn on wifi. bit1: 1. Other bits default to 0; |
|  | SSID | 32 | Target WiFi name, Asic code. Default: administrator, invalid bytes are filled with 0x00 |
|  | PWD | 64 | Target WiFi password, Asic code. Default: 12345678, invalid bytes are padded with 0x00 |
|  | RemoteIPAddr | 4 | Remote IP address, if the IP address is 192.168.1.100, the data is [0xC0,A8,0x01,0x64] |
|  | RemotePort | 2 | The remote IP port number, with the high byte first and the low byte last. Default: 0x1388 (5000) |

2.2.13 RFM_SET_GET_S-PERMISSION_PARAM (Set read permission)

- This command is used to set the password and mask query permission parameters of a specific tag.

- Command format

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0073 | N Byte | 1Byte | 1Byte | 4Bytes | 1Byte | 1Byte | 1Byte | 31Bytes | 1Byte | 2Bytes |

  - Option: command control option, 0x01 is set, 0x02 is read, other values are invalid

  - CodeEN: Password function enable parameter, length 1 byte. 0x01 is enabled, 0x00 is not enabled. Default 0x00.

  - Codes    ：The access password of the tag, with a length of 4 bytes. Default [0x00,0x00,0x00,0x00].

  - MaskEN：Mask function enable parameter, the length is 1 byte. 0x01 is enabled, 0x00 is not enabled. Default 0x00.

  - StartAdd：Mask start address, length 1 byte, unit byte. Default 0x00.

  - Mask Len：Mask length, length 1 byte, unit byte, maximum 31. Default 0x00.

  - MaskData：Mask data, length 31 bytes, when the mask is less than 31 bytes, the following byte data is supplemented with 0.

  - The default is all 0.

  - MaskCondition：Mask condition, length 1 byte. 0x00: Password or mask match; 0x01: Password and mask match at the same time. Default 0x00.

- Response format and status byte

- Set response:

| HEAD | ADDR | CMD | LEN | STATUS | Option | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0073 | 1Byte | 1Byte | 1Byte | 2Byte |

- Get the response:

| HEAD | Control Field |  |  | PAYLOAD |  |  |  |  |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADDR | CMD | LEN | STATUS | OPTION | Code EN | Codes | Mask EN | Start Add | Mask Len | Mask Data | Mask Condition |  |
| 0xCF | 0xFF | 0x0073 | N Byte | 1Byte | 1Byte | 1Byte | 4Byte | 1Byte | 1Byte | 1Byte | 31Byte | 1Byte | 2Bytes |

  - STATUS: 0x00 is executed successfully, other values are shown in Table A-6;

2.3 International standard (ISO 18000-6C) protocol related commands

2.3.1 RFM_INVENTORYISO_CONTINUE (Label inventory)

- This command is to start the international standard multi-label inventory command, and the inventory function has an anti-collision algorithm.

- This command can specify the counting time. If the input counting times is 0, it means that the counting label will continue to be counted until the stop counting command is received. During the inventory process, each time a tag is successfully inventoryed, the newly inventoryed tag information will be returned through a command response with a "STATUS" value of "0".

- When the inventory command is successfully executed, a command response with a "STATUS" value of "0x12" will be returned to notify that the inventory command has been executed.

- Command format

| HEAD | ADDR | CMD | LEN | PAYLOAD |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | InvType | InvParam |  |
| 0xCF | 0xFF | 0x0001 | 0x05 | 1Byte | 4Byte | 2Byte |

InvType: Inventory method:

0x00: according to the time inventory label, stop inventory after executing the specified time or stop inventory after receiving the stop inventory command;

0x01: Counting according to the number of cycles, stop counting after executing the specified number of polls or receiving the stop counting instruction;

InvParam: Inventory method parameters:

1. If InvType is 0x00:

InvParam represents the counting time, the unit is: seconds, if the value is 0, it means that the counting label is continued until the stop counting command is received;

2. If InvType is 0x01: InvParamIndicates the number of counts, the unit is: times, the value must be greater than 0;

- Response format and status bytes

| HEAD | ADDR | CMD | LEN | STATUS | PAYLOAD |  |  |  |  | CHECK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | RSSI | Antenna | Channel | EPC LEN | EPC NUM |  |
| 0xCF | 0x00 | 0x0001 | N Byte | 1Byte | 2Bytes | 1 Byte | 1 Byte | 1Byte | N Bytes | 2Byte |

Definition of each byte:

| field | byte | describe |
| --- | --- | --- |
| STATUS | 1 | 0x00 is executed successfully, other values are shown in Table A-6; |
| RSSI | 2 | RSSI of tag ACK response, unit is dBm, signed number, negative number uses complement format; |
| Channel | 1 | From which channel the tag data is received, the value starts from 0, 0 means 0 channel, 1 means 1 channel, and so on; |
| EPC LEN | 1 | Label's EPC number length (bytes); |
| EPC NUM | N Byte | EPC number of the label; |

2.3.2 RFM_INVENTORY_STOP (stop counting)

This command user to actively stop the international standard multi-label anti-collision inventory process.

- command format

| HEAD | ADDR | CMD | LEN | CHECK |
| --- | --- | --- | --- | --- |
| 0xCF | 0xFF | 0x0002 | 0x00 | 2Byte |

- Response format and status bytes

| HEAD | ADDR | CMD | LEN | STATUS | CHECK |
| --- | --- | --- | --- | --- | --- |
| 0xCF | 0x00 | 0x0002 | 0x01 | 1Byte | 2Byte |

 STATUS：This command will only return successful execution (value 0x00), other values are invalid;

Appendix A. Operational Status Returned by the Tag

The operation status code returned by the tag has a total of 8 bits, see Table A-1.

Table A-1 Operational Status Returned by the Tag

| Operational Status Coding | operating state | describe | Error priority |
| --- | --- | --- | --- |
| 0x81 | other errors | Unknown error returned by label |  |
| 0x82 | bucket overflow | target store does not exist |  |
| 0x83 | store lock | Write to a memory area that is locked as unwritable |  |
| 0x84 | Insufficient power | operation or erase operation, read operation to the memory area that is locked as unreadable |  |
| 0x85 | non-specific error | The tag does not have enough energy to complete the operation |  |

Appendix B. CRC16 check reference C code

as follows：

#define PRESET_VALUE 0xFFFF

#define POLYNOMIAL  0x8408

unsigned int uiCrc16Cal(unsigned char const  * pucY, unsigned char ucX)

{

unsigned char ucI,ucJ;

unsigned short int  uiCrcValue = PRESET_VALUE;

for(ucI = 0; ucI < ucX; ucI++)

{

uiCrcValue = uiCrcValue ^ *(pucY + ucI);

for(ucJ = 0; ucJ < 8; ucJ++)

{

if(uiCrcValue & 0x0001)

{

uiCrcValue = (uiCrcValue >> 1) ^ POLYNOMIAL;

}

else

{

uiCrcValue = (uiCrcValue >> 1);

}

}

}

return uiCrcValue;

}

Appendix C. Definition of STATUS。

Table A-6 STATUS Definitions

| STATUS | wrong description |
| --- | --- |
| 0x00 | The execution is successful (here only means that the module successfully receives the tag response data. If there is a tag execution status in the tag response, it should be further judged whether the tag execution status is correct) |
| 0x01 | The parameter value is wrong or out of bounds, or the parameter value is not supported by the module |
| 0x02 | Command execution failed due to module internal error |
| 0x03 | reserve |
| 0x12 | No count to label or complete count command execution |
| 0x14 | Tag response timed out |
| 0x15 | Demod tag response error |
| 0x16 | Protocol authentication failed |
| 0x17 | wrong password |
| 0xFF | no more data |